Configuring Router as DHCP LAB

Topology 
• Same topology as NAT Overload (PAT) lab
• 1x Router — R1 (DHCP Server)
• 1x Switch (2960)
• 3x PCs — PC1, PC2, PC3
• 1x Server on R2 side (reachable through the existing routing/NAT setup)

- Prepare PCs to use DHCP (Desktop > IP Configuration)
• On PC1: select DHCP
• On PC2: select DHCP
• On PC3: select DHCP

- Exclude gateway IP on R1 (so DHCP does not assign it)

R1

R1> enable

R1# configure terminal

R1(config)# ip dhcp excluded-address 192.168.1.1

- Create DHCP pool on R1 (LAN-POOL)

R1

R1(config)# ip dhcp pool LAN-POOL

R1(dhcp-config)# network 192.168.1.0 255.255.255.0

R1(dhcp-config)# default-router 192.168.1.1

R1(dhcp-config)# dns-server 8.8.8.8

R1(dhcp-config)# end

- Verify PCs received IP addresses from DHCP
PC1 (Desktop > IP Configuration)
• Expected: 192.168.1.2 /24, Gateway 192.168.1.1

PC2 (Desktop > IP Configuration)
• Expected: next available IP from the pool (example 192.168.1.3)

PC3 (Desktop > IP Configuration)
• Expected: next available IP from the pool (example 192.168.1.4)

- Verify DHCP details using ipconfig

PC1 (Command Prompt)

PC1> ipconfig

PC1> ipconfig /all

- Test connectivity to Server (ping)

PC1 (Command Prompt)

PC1> ping 200.1.1.2

PC2 (Command Prompt)

PC2> ping 200.1.1.2

PC3 (Command Prompt)

PC3> ping 200.1.1.2

End of Lab Output Expectations
• R1 assigns IP addresses automatically to PC1, PC2, and PC3.
• Gateway IP (192.168.1.1) is never assigned to a PC.
• PCs receive subnet mask, default gateway, and DNS server from the pool.
• All PCs can ping the server successfully after getting valid DHCP leases.