Dynamic NAT Configuration LAB

Topology  
• 2x Routers (2911) — R1 and R2
• 1x Switch (2960) — LAN switch for R1 side
• 2x PCs — PC1 and PC2 (LAN behind R1)
• 1x Server — Server (LAN behind R2)
• R1 ↔ R2 connected using Serial0/3/0 to Serial0/3/0

- Verify Static NAT is working (before changes)

R1

R1# show ip nat translations

PC1 (Command Prompt)

PC1> ping 200.1.1.2

R1

R1# show ip nat translations

- Update topology for Dynamic NAT
• Remove the direct cable between R1 and the PC
• Add 1 switch (2960)
• Rename existing PC to PC1
• Add a second PC (PC2)

Cable the devices (Copper Straight-Through)
Connect these ports:
• Switch Gi0/1 → R1 Gi0/0
• PC1 Fa0 → Switch Fa0/1
• PC2 Fa0 → Switch Fa0/2

- Configure IP addressing on PCs

PC1 (Desktop > IP Configuration)
• IP: 192.168.1.2
• Mask: 255.255.255.0
• Gateway: 192.168.1.1

PC2 (Desktop > IP Configuration)
• IP: 192.168.1.3
• Mask: 255.255.255.0
• Gateway: 192.168.1.1

- Change Serial interface subnet mask to /24 (both routers)

R1

R1# show running-config

R1# configure terminal

R1(config)# interface serial0/3/0

R1(config-if)# ip address 20.0.0.1 255.255.255.0

R1(config-if)# end

R2

R2# configure terminal

R2(config)# interface serial0/3/0

R2(config-if)# ip address 20.0.0.2 255.255.255.0

R2(config-if)# end

- Remove Static NAT configuration on R1

R1 (remove the old static NAT mapping)

R1# show running-config

R1# configure terminal

R1(config)# no ip nat inside source static 192.168.1.2 20.0.0.1

R1(config)# end

Verify NAT table is empty

R1# show ip nat translations

- Configure Dynamic NAT on R1 (Pool + ACL + NAT rule)

R1 (create pool)

R1# configure terminal

R1(config)# ip nat pool DYN-POOL 20.0.0.10 20.0.0.20 netmask 255.255.255.0

R1 (create ACL for inside network)

R1(config)# access-list 1 permit 192.168.1.0 0.0.0.255

R1 (bind ACL to pool)

R1(config)# ip nat inside source list 1 pool DYN-POOL

R1(config)# end

Verify configuration

R1# show running-config

- Test Dynamic NAT (generate traffic)

PC1 (Command Prompt)

PC1> ping 200.1.1.2

PC2 (Command Prompt)

PC2> ping 200.1.1.2

- Verify NAT translations

R1

R1# show ip nat translations

End of Lab Output Expectations
• PC1 and PC2 can ping Server (200.1.1.2) successfully.
• R1 dynamically assigns inside hosts public IPs from 20.0.0.10–20.0.0.20.
• “show ip nat translations” shows inside local (192.168.1.2 / 192.168.1.3) mapped to inside global (20.0.0.10 / 20.0.0.11 etc.).