NTP Concept and Configuration LAB

Topology  
• Use the same topology from NAT Overload (PAT) lab
• 2x Routers — R1 and R2
• Serial link between routers:
• R1 Serial0/3/0 = 20.0.0.1
• R2 Serial0/3/0 = 20.0.0.2

- Verify current time on both routers (unsynchronized)

R1

R1> enable

R1# show clock

R2

R2> enable

R2# show clock

- Verify connectivity between routers

R1

R1# ping 20.0.0.2

- Manually set time on R1 and configure timezone

R1 (set clock)

R1# clock set 02:29:00 18 October 2025

R1# show clock

R1 (set timezone to GMT)

R1# configure terminal

R1(config)# clock timezone GMT 0

R1(config)# end

R1# show clock

- Configure R1 as NTP Master (Stratum 1)

R1

R1# configure terminal

R1(config)# ntp master 1

R1(config)# end

- Configure R2 as NTP Client (sync from R1)

R2

R2# configure terminal

R2(config)# ntp server 20.0.0.1

R2(config)# end

- Verify NTP synchronization on R2

R2 (associations + status)

R2# show ntp associations

R2# show ntp status

Wait 1–2 minutes, then run again:

R2# show ntp status

R2# show ntp associations

- Verify time is synchronized (check clock on both routers)

R1

R1# show clock

R2

R2# show clock

End of Lab Output Expectations
• R1 clock is manually set and shows GMT time.
• R1 acts as NTP master (stratum 1).
• R2 syncs time from R1 using NTP.
• “show ntp status” on R2 shows synchronized after some time.
• “show clock” on R2 matches R1 closely, confirming synchronization.