NAT Oveload PAT Configuration LAB

Topology 
• Same topology as Dynamic NAT Lab
• 2x Routers (2911) — R1 and R2
• 1x Switch (2960) — LAN switch for R1 side
• 2x PCs — PC1 and PC2 (LAN behind R1)
• 1x Server — Server (LAN behind R2)
• Later: Add PC3 (LAN behind R1)

- Configure PAT (NAT Overload) using the Serial interface IP

R1

Router(config)#int Gig0/0/0

Router(config-if)#ip nat inside

Router(config-if)#exit

Router(config)#int Gig0/0/1

Router(config-if)#ip nat outside

Router(config-if)#exit

Router(config)#access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)#ip nat inside source list interface Gig0/0/1 overload

- Test PAT from PC1 and PC2

PC1 (Command Prompt)

PC1> ping 192.168.2.2

PC2 (Command Prompt)

PC2> ping 192.168.2.2

- Verify NAT translations (observe port numbers)

R1

R1# show ip nat translations

What to observe
• Inside local shows 192.168.1.2 and 192.168.1.3
• Inside global is the same public IP (Gig0/0/1)
• Port numbers are different for each session (this is PAT)

- Add PC3 and test PAT with 3 hosts

Cable PC3 (Copper Straight-Through)
• PC3 Fa0 → Switch Fa0/3

PC3 (Desktop > IP Configuration)
• IP: 192.168.1.4
• Mask: 255.255.255.0

‘• Gateway: 192.168.1.1

PC3 (Command Prompt)

PC3> ping 192.168.2.2

- Verify NAT translations again (PC3 appears with same public IP + new port)

R1

R1# show ip nat translations

- Generate more traffic and re-check translations

PC2 (Command Prompt)

PC2> ping 192.168.2.2

R1

R1# show ip nat translations

End of Lab Output Expectations
• PC1, PC2, and PC3 can ping Server (192.168.2.2).
• All inside local IPs (192.168.1.2 / .3 / .4) translate to the same inside global IP (Serial0/3/0).
• NAT table shows different port numbers per session, confirming PAT/NAT Overload is working.