# Static NAT Configuration LAB

Topology
• 2x Routers (2911) — Router1 (R1), Router2 (R2)
• 1x PC (LAN behind R1)
• 1x Server (LAN behind R2)
• R1 ↔ R2 connected using Serial link (Serial0/3/0 to Serial0/3/0)

- Add serial module (HWIC-2T) on both routers
Router1 (Physical Tab)
• Power OFF router
• Insert HWIC-2T into an empty slot
• Power ON router

Router2 (Physical Tab)
• Power OFF router
• Insert HWIC-2T into an empty slot
• Power ON router

- Verify serial interfaces exist

Router1

Router1> enable

Router1# show ip interface brief

Router2

Router2> enable

Router2# show ip interface brief

- Connect routers using Serial DCE cable
• Use Serial DCE cable
• Connect R1 Serial0/3/0 → R2 Serial0/3/0
• Clock rate will be configured on DCE side (R1) as 64000

- Configure Router1 (R1)

R1

Router1# configure terminal

Router1(config)# hostname R1

R1(config)# interface gigabitEthernet0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface serial0/3/0

R1(config-if)# ip address 20.0.0.1 255.255.255.252

R1(config-if)# no shutdown

R1(config-if)# clock rate 64000

R1(config-if)# end

- Configure Router2 (R2)

R2

Router2# configure terminal

Router2(config)# hostname R2

R2(config)# interface serial0/3/0

R2(config-if)# ip address 20.0.0.2 255.255.255.252

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# interface gigabitEthernet0/0

R2(config-if)# ip address 200.1.1.1 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# end

- Configure IP addresses on PC and Server

PC (Desktop > IP Configuration)
• IP: 192.168.1.2
• Mask: 255.255.255.0
• Gateway: 192.168.1.1

Server (Desktop > IP Configuration)
• IP: 200.1.1.2
• Mask: 255.255.255.0
• Gateway: 200.1.1.1

- Configure OSPF on both routers (so routes are learned automatically)

R1

R1# configure terminal

R1(config)# router ospf 10

R1(config-router)# router-id 1.1.1.1

R1(config-router)# network 192.168.1.0 0.0.0.255 area 0

R1(config-router)# network 20.0.0.0 0.0.0.3 area 0

R1(config-router)# end

R2

R2# configure terminal

R2(config)# router ospf 10

R2(config-router)# router-id 2.2.2.2

R2(config-router)# network 20.0.0.0 0.0.0.3 area 0

R2(config-router)# network 200.1.1.0 0.0.0.255 area 0

R2(config-router)# end

- Verify routing and end-to-end connectivity (before NAT)

R2

R2# show ip route

R1

R1# show ip route

PC (Command Prompt)

PC> ping 200.1.1.2

- Verify NAT table before configuration (should be empty)

R1

R1# show ip nat translations

- Configure Static NAT on R1

R1

R1# configure terminal

R1(config)# ip nat inside source static 192.168.1.2 20.0.0.1

R1(config)# interface gigabitEthernet0/0

R1(config-if)# ip nat inside

R1(config-if)# exit

R1(config)# interface serial0/3/0

R1(config-if)# ip nat outside

R1(config-if)# exit

R1(config)# end

- Verify NAT translations after configuration

R1

R1# show ip nat translations

- Test NAT by generating traffic and verify translations

PC (Command Prompt)

PC> ping 200.1.1.2

R1

R1# show ip nat translations

End of Lab Output Expectations
• Serial link is up between R1 and R2 (clock rate set on DCE side).
• OSPF learns routes on both routers (LANs reachable).
• Static NAT mapping exists: 192.168.1.2 ↔ 20.0.0.1.
• PC can ping Server successfully.
• “show ip nat translations” shows inside local, inside global, and outside addresses after traffic.