# OSPF DR-BDR Election LAB

Topology
• 3x Routers: RouterA, RouterB, RouterC
• 1x Switch: Switch1
• 1x PC: PC-A
• All three routers connected to Switch1 on the same subnet (Broadcast network)

Addressing
• RouterA G0/0: 192.168.10.1/24
• RouterB G0/0: 192.168.10.2/24
• RouterC G0/0: 192.168.10.3/24
• PC-A: 192.168.10.10/24, GW 192.168.10.1

- Cable the devices (Copper Straight-Through)
Connect these ports:
• RouterA G0/0 → Switch1 Fa0/1
• RouterB G0/0 → Switch1 Fa0/2
• RouterC G0/0 → Switch1 Fa0/3
• PC-A Fa0 → Switch1 Fa0/4

- Configure IP addressing on routers (G0/0)

RouterA

Router> enable
Router# configure terminal
Router(config)# hostname RouterA
RouterA(config)# interface gigabitEthernet0/0
RouterA(config-if)# ip address 192.168.10.1 255.255.255.0
RouterA(config-if)# no shutdown
RouterA(config-if)# exit
RouterA(config)# end
RouterA# show ip interface brief

RouterB

Router> enable
Router# configure terminal
Router(config)# hostname RouterB
RouterB(config)# interface gigabitEthernet0/0
RouterB(config-if)# ip address 192.168.10.2 255.255.255.0
RouterB(config-if)# no shutdown
RouterB(config-if)# exit
RouterB(config)# end
RouterB# show ip interface brief

RouterC

Router> enable
Router# configure terminal
Router(config)# hostname RouterC
RouterC(config)# interface gigabitEthernet0/0
RouterC(config-if)# ip address 192.168.10.3 255.255.255.0
RouterC(config-if)# no shutdown
RouterC(config-if)# exit
RouterC(config)# end
RouterC# show ip interface brief

What to observe
• On all routers, G0/0 should show Status up and Protocol up.

- Configure IP addressing on PC-A

PC-A (Desktop > IP Configuration)
• IP: 192.168.10.10
• Mask: 255.255.255.0
• Gateway: 192.168.10.1

- Enable OSPF and set Router IDs (Area 0)

RouterA

RouterA# configure terminal
RouterA(config)# router ospf 1
RouterA(config-router)# router-id 1.1.1.1
RouterA(config-router)# network 192.168.10.0 0.0.0.255 area 0
RouterA(config-router)# end

RouterB

RouterB# configure terminal
RouterB(config)# router ospf 1
RouterB(config-router)# router-id 2.2.2.2
RouterB(config-router)# network 192.168.10.0 0.0.0.255 area 0
RouterB(config-router)# end

RouterC

RouterC# configure terminal
RouterC(config)# router ospf 1
RouterC(config-router)# router-id 3.3.3.3
RouterC(config-router)# network 192.168.10.0 0.0.0.255 area 0
RouterC(config-router)# end

What to observe
• Neighbor adjacency forms and moves through states until FULL.

- Verify neighbors and DR/BDR election (initial election)

Run on RouterA

RouterA# show ip ospf neighbor
RouterA# show ip ospf interface gigabitEthernet0/0

What to observe
• Two neighbors should appear (2.2.2.2 and 3.3.3.3) in FULL state.
• The DR/BDR you see may not match “highest router-id wins” if routers were configured one-by-one.
• Once DR/BDR are elected, they stay until OSPF is reset or the interface goes down.

- Force RouterC to become DR using OSPF interface priority

Set higher priority on RouterC (G0/0)

RouterC# configure terminal
RouterC(config)# interface gigabitEthernet0/0
RouterC(config-if)# ip ospf priority 100
RouterC(config-if)# end

Reset OSPF process on all routers (to trigger a fresh election)

RouterA

RouterA# clear ip ospf process

RouterB

RouterB# clear ip ospf process

RouterC

RouterC# clear ip ospf process

When prompted: Reset ALL OSPF processes? → type yes

- Verify DR/BDR election after priority change

On RouterB

RouterB# show ip ospf neighbor
RouterB# show ip ospf interface gigabitEthernet0/0

On RouterA

RouterA# show ip ospf neighbor

What to observe
• RouterC (3.3.3.3) should now be DR because priority 100 is higher than default 1.
• RouterB should become BDR.
• RouterA should be DROTHER.

- Test connectivity (same subnet reachability)

PC-A (Desktop > Command Prompt)

ping 192.168.10.1
ping 192.168.10.2
ping 192.168.10.3

End of Lab Output Expectations
• All routers form FULL adjacency on the broadcast segment.
• Initial DR/BDR may reflect startup order.
• After setting RouterC priority to 100 and clearing OSPF, RouterC becomes DR, RouterB becomes BDR, RouterA becomes DROTHER.
• PC-A can ping all three router interfaces successfully.