# Floating Static Route Lab (Packet Tracer)

Topology
• 2x Routers: RouterA, RouterB
• 2x PCs: PC-A, PC-B
• Primary link: RouterA G0/1 ↔ RouterB G0/1 (10.0.0.0/30)
• Backup link: RouterA G0/2 ↔ RouterB G0/2 (10.0.1.0/30)
• LANs:
• PC-A LAN: 192.168.1.0/24 (Gateway 192.168.1.1)
• PC-B LAN: 192.168.2.0/24 (Gateway 192.168.2.1)

- Configure RouterA interfaces

RouterA

Router> enable
Router# configure terminal
Router(config)# hostname RouterA

RouterA(config)# interface gigabitEthernet0/0
RouterA(config-if)# ip address 192.168.1.1 255.255.255.0
RouterA(config-if)# no shutdown
RouterA(config-if)# exit

RouterA(config)# interface gigabitEthernet0/1
RouterA(config-if)# ip address 10.0.0.1 255.255.255.252
RouterA(config-if)# no shutdown
RouterA(config-if)# exit

RouterA(config)# interface gigabitEthernet0/2
RouterA(config-if)# ip address 10.0.1.1 255.255.255.252
RouterA(config-if)# no shutdown
RouterA(config-if)# end

- Configure RouterB interfaces

RouterB

Router> enable
Router# configure terminal
Router(config)# hostname RouterB

RouterB(config)# interface gigabitEthernet0/0
RouterB(config-if)# ip address 192.168.2.1 255.255.255.0
RouterB(config-if)# no shutdown
RouterB(config-if)# exit

RouterB(config)# interface gigabitEthernet0/1
RouterB(config-if)# ip address 10.0.0.2 255.255.255.252
RouterB(config-if)# no shutdown
RouterB(config-if)# exit

RouterB(config)# interface gigabitEthernet0/2
RouterB(config-if)# ip address 10.0.1.2 255.255.255.252
RouterB(config-if)# no shutdown
RouterB(config-if)# end

- Configure primary + floating static routes

RouterA (to reach 192.168.2.0/24)

RouterA# configure terminal
RouterA(config)# ip route 192.168.2.0 255.255.255.0 10.0.0.2
RouterA(config)# ip route 192.168.2.0 255.255.255.0 10.0.1.2 200
RouterA(config)# end

RouterB (to reach 192.168.1.0/24)

RouterB# configure terminal
RouterB(config)# ip route 192.168.1.0 255.255.255.0 10.0.0.1
RouterB(config)# ip route 192.168.1.0 255.255.255.0 10.0.1.1 200
RouterB(config)# end

- Configure PCs

PC-A (Desktop > IP Configuration)
• IP: 192.168.1.10
• Mask: 255.255.255.0
• Gateway: 192.168.1.1

PC-B (Desktop > IP Configuration)
• IP: 192.168.2.10
• Mask: 255.255.255.0
• Gateway: 192.168.2.1

- Verify routing tables (primary route active)

RouterA

RouterA# show ip route

RouterB

RouterB# show ip route

Expected
• Only the primary static route (AD 1) is active in the routing table
• The floating route (AD 200) stays as a backup and doesn’t appear while primary is up

- Test connectivity and confirm path (primary link)

PC-A (Command Prompt)

PC-A> ping 192.168.2.10
PC-A> tracert 192.168.2.10

Expected tracert hops
• 192.168.1.1
• 10.0.0.2
• 192.168.2.10

- Simulate failover (shut primary link)

RouterA

RouterA# configure terminal
RouterA(config)# interface gigabitEthernet0/1
RouterA(config-if)# shutdown
RouterA(config-if)# end

Verify routes switched to backup

RouterA# show ip route
RouterB# show ip route

Expected
• Route now uses next-hop 10.0.1.2 / 10.0.1.1 (floating route becomes active)

- Confirm path after failover (backup link)

PC-A (Command Prompt)

PC-A> tracert 192.168.2.10

Expected tracert hops
• 192.168.1.1
• 10.0.1.2
• 192.168.2.10

- Restore primary link and verify it becomes preferred again

RouterA

RouterA# configure terminal
RouterA(config)# interface gigabitEthernet0/1
RouterA(config-if)# no shutdown
RouterA(config-if)# end

Verify primary is active again

RouterA# show ip route

End of Lab Output Expectations
• Primary static route (AD 1) is used when the primary link is up.
• When the primary link fails, the floating route (AD 200) becomes active automatically.
• When the primary link returns, the router switches back to the primary route automatically.