# Routing Configuration Lab – Directly Connected Networks

Topology
• 3x Routers: RouterA, RouterB, RouterC
• Point-to-point links:
• RouterA ↔ RouterB (10.1.1.0/30)
• RouterB ↔ RouterC (10.2.2.0/30)
• LANs:
• RouterA LAN: 192.168.1.0/24 (PC A)
• RouterC LAN: 192.168.2.0/24 (PC B)

- Configure RouterA interfaces (Directly connected networks)

RouterA

Router> enable
Router# configure terminal
Router(config)# hostname RouterA

RouterA(config)# interface gigabitEthernet0/0
RouterA(config-if)# ip address 10.1.1.1 255.255.255.252
RouterA(config-if)# no shutdown
RouterA(config-if)# exit

RouterA(config)# interface gigabitEthernet0/1
RouterA(config-if)# ip address 192.168.1.1 255.255.255.0
RouterA(config-if)# no shutdown
RouterA(config-if)# end

- Configure RouterB interfaces (Directly connected networks)

RouterB

Router> enable
Router# configure terminal

Router(config)# interface gigabitEthernet0/0
Router(config-if)# ip address 10.1.1.2 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1
Router(config-if)# ip address 10.2.2.1 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# hostname RouterB
RouterB(config)# end

Verify RouterB interfaces

RouterB# show ip interface brief

- Configure RouterC interfaces (Directly connected networks)

RouterC

Router> enable
Router# configure terminal

Router(config)# interface gigabitEthernet0/0
Router(config-if)# ip address 10.2.2.2 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1
Router(config-if)# ip address 192.168.2.1 255.255.255.0
Router(config-if)# no shutdown
Router(config-if)# end

Verify RouterC interfaces

RouterC# show ip interface brief

- Configure PCs

PC A (Desktop > IP Configuration)
• IP: 192.168.1.10
• Mask: 255.255.255.0
• Gateway: 192.168.1.1

PC B (Desktop > IP Configuration)
• IP: 192.168.2.10
• Mask: 255.255.255.0
• Gateway: 192.168.2.1

- Test router-to-router connectivity (direct neighbors)

RouterA

RouterA# ping 10.1.1.2

RouterB

RouterB# ping 10.2.2.2

- View routing table (shows only directly connected routes)

RouterB

RouterB# show ip route

What to observe
• Routes marked with C are directly connected networks (learned automatically).
• RouterB should show: 10.1.1.0/30 and 10.2.2.0/30 as connected routes.

- Test end-to-end connectivity (should fail)

PC A (Command Prompt)

PC-A> ping 192.168.2.10

Expected result
• Ping fails because routers only know their directly connected networks.
• No static routes or routing protocol exists to reach 192.168.2.0/24 from RouterA and 192.168.1.0/24 from RouterC.

End of Lab Output Expectations
• Each router automatically installs connected routes for its configured interfaces.
• Router-to-router pings work only between directly connected neighbors.
• PC-A cannot reach PC-B until static routes or a dynamic routing protocol is added.