# IPv4 Static Route Configuration Lab

Topology
• 3x Routers in triangle: RouterA, RouterB, RouterC
• 3x Hosts: PC-A, PC-B, PC-C
• LANs:
• PC-A: 192.168.1.0/24 (Gateway 192.168.1.1)
• PC-B: 192.168.2.0/24 (Gateway 192.168.2.1)
• PC-C: 192.168.3.0/24 (Gateway 192.168.3.1)
• Point-to-point links (/30):
• A ↔ B: 10.1.1.0/30
• A ↔ C: 10.2.2.0/30
• B ↔ C: 10.3.3.0/30

- Configure RouterA interfaces

RouterA

Router> enable
Router# configure terminal

Router(config)# interface gigabitEthernet0/0
Router(config-if)# ip address 192.168.1.1 255.255.255.0
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1
Router(config-if)# ip address 10.1.1.1 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/2
Router(config-if)# ip address 10.2.2.1 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# end

- Configure RouterB interfaces

RouterB

Router> enable
Router# configure terminal

Router(config)# interface gigabitEthernet0/0
Router(config-if)# ip address 192.168.2.1 255.255.255.0
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1
Router(config-if)# ip address 10.1.1.2 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/2
Router(config-if)# ip address 10.3.3.1 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# end

- Configure RouterC interfaces

RouterC

Router> enable
Router# configure terminal

Router(config)# interface gigabitEthernet0/0
Router(config-if)# ip address 192.168.3.1 255.255.255.0
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1
Router(config-if)# ip address 10.2.2.2 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/2
Router(config-if)# ip address 10.3.3.2 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# end

- Verify interfaces are up

On each router

show ip interface brief

- Configure IPv4 static routes

RouterA (reach PC-B and PC-C LANs)

RouterA# configure terminal
RouterA(config)# ip route 192.168.2.0 255.255.255.0 10.1.1.2
RouterA(config)# ip route 192.168.3.0 255.255.255.0 10.2.2.2
RouterA(config)# end
RouterA# show ip route

RouterB (reach PC-A and PC-C LANs)

RouterB# configure terminal
RouterB(config)# ip route 192.168.1.0 255.255.255.0 10.1.1.1
RouterB(config)# ip route 192.168.3.0 255.255.255.0 10.3.3.2
RouterB(config)# end
RouterB# show ip route

RouterC (reach PC-A and PC-B LANs)

RouterC# configure terminal
RouterC(config)# ip route 192.168.1.0 255.255.255.0 10.2.2.1
RouterC(config)# ip route 192.168.2.0 255.255.255.0 10.3.3.1
RouterC(config)# end
RouterC# show ip route

- Configure IP addresses on PCs

PC-A (Desktop > IP Configuration)
• IP: 192.168.1.10
• Mask: 255.255.255.0
• Gateway: 192.168.1.1

PC-B (Desktop > IP Configuration)
• IP: 192.168.2.10
• Mask: 255.255.255.0
• Gateway: 192.168.2.1

PC-C (Desktop > IP Configuration)
• IP: 192.168.3.10
• Mask: 255.255.255.0
• Gateway: 192.168.3.1

- Test full connectivity (pings)

PC-A (Command Prompt)

PC-A> ping 192.168.2.10
PC-A> ping 192.168.3.10

PC-B (Command Prompt)

PC-B> ping 192.168.1.10
PC-B> ping 192.168.3.10

PC-C (Command Prompt)

PC-C> ping 192.168.1.10
PC-C> ping 192.168.2.10

- Troubleshooting check (if any ping fails)

On the router facing the issue

show running-config
show ip route

- Simulate link failure (shut down A ↔ C link)

RouterA

RouterA# configure terminal
RouterA(config)# interface gigabitEthernet0/2
RouterA(config-if)# shutdown
RouterA(config-if)# end

PC-A test to PC-C (expected fail)

PC-A> ping 192.168.3.10

- Reroute traffic via RouterB (manual static route update)

RouterA (send 192.168.3.0/24 via RouterB)

RouterA# configure terminal
RouterA(config)# ip route 192.168.3.0 255.255.255.0 10.1.1.2
RouterA(config)# end

RouterC (send 192.168.1.0/24 via RouterB)

RouterC# configure terminal
RouterC(config)# ip route 192.168.1.0 255.255.255.0 10.3.3.1
RouterC(config)# end

Test again (now should work through RouterB)

PC-A> ping 192.168.3.10

End of Lab Output Expectations
• Each router shows static routes (S) for remote LANs in “show ip route”.
• All PCs can ping each other when all links are up.
• When A–C link is shut, PC-A to PC-C fails until static routes are manually updated to use RouterB.
• Static routes do not adapt automatically; changes must be done manually on both sides for return traffic.