# OSPF Process ID Redistribution Lab

Topology
• 2x Routers: RouterA, RouterB
• Point-to-Point link: RouterA G0/0 ↔ RouterB G0/0 (10.0.0.0/30)
• Existing LANs (from previous lab)
– RouterA G0/1: 192.168.1.0/24 (PC-A)
– RouterB G0/1: 192.168.2.0/24 (PC-B)
• New LANs (this lab)
– RouterA G0/2: 192.168.10.0/24 (PC-C)
– RouterB G0/2: 192.168.20.0/24 (PC-D)

- Add new PCs and cable

Cable (Copper Straight-Through)
• RouterA G0/2 → PC-C Fa0
• RouterB G0/2 → PC-D Fa0

Rename
• PC-C, PC-D

- Configure RouterA new LAN interface (G0/2)

RouterA

RouterA> enable
RouterA# configure terminal
RouterA(config)# interface gigabitEthernet0/2
RouterA(config-if)# ip address 192.168.10.1 255.255.255.0
RouterA(config-if)# no shutdown
RouterA(config-if)# exit
RouterA(config)# end

- Configure RouterB new LAN interface (G0/2)

RouterB

RouterB> enable
RouterB# configure terminal
RouterB(config)# interface gigabitEthernet0/2
RouterB(config-if)# ip address 192.168.20.1 255.255.255.0
RouterB(config-if)# no shutdown
RouterB(config-if)# exit
RouterB(config)# end

- Create additional OSPF processes (independent processes)

RouterA (OSPF process 10 for 192.168.10.0/24)

RouterA# configure terminal
RouterA(config)# router ospf 10
RouterA(config-router)# network 192.168.10.0 0.0.0.255 area 0
RouterA(config-router)# end

RouterB (OSPF process 20 for 192.168.20.0/24)

RouterB# configure terminal
RouterB(config)# router ospf 20
RouterB(config-router)# network 192.168.20.0 0.0.0.255 area 0
RouterB(config-router)# end

What to observe
• Process IDs do NOT need to match across routers.
• These new processes do NOT form adjacency on the 10.0.0.0/30 link (because that link is still under process 1 from the previous lab).
• Each process has its own LSDB and routes stay separate unless redistributed.

- Verify OSPF processes exist

RouterA

RouterA# show ip ospf

RouterB

RouterB# show ip ospf

What to observe
• You should see two processes on each router (RouterA: ospf 1 and ospf 10, RouterB: ospf 1 and ospf 20).
• show ip ospf shows processes + router IDs (not the networks list).

- Verify routing table BEFORE redistribution

RouterA

RouterA# show ip route

RouterB

RouterB# show ip route

What to observe
• You will NOT see routes to 192.168.10.0/24 or 192.168.20.0/24 learned through process 1.
• This proves the OSPF processes are independent (blind to each other).

- Redistribute between processes (inject routes into process 1)

RouterA: redistribute process 10 into process 1

RouterA# configure terminal
RouterA(config)# router ospf 1
RouterA(config-router)# redistribute ospf 10 subnets
RouterA(config-router)# end

RouterB: redistribute process 20 into process 1

RouterB# configure terminal
RouterB(config)# router ospf 1
RouterB(config-router)# redistribute ospf 20 subnets
RouterB(config-router)# end

What this does
• OSPF 1 will advertise the redistributed networks as external routes (E2 by default).

- Verify routing table AFTER redistribution

RouterA

RouterA# show ip route

RouterB

RouterB# show ip route

What to observe
• Routes now appear as O E2 (OSPF External Type 2).
• RouterA should learn 192.168.20.0/24 as O E2.
• RouterB should learn 192.168.10.0/24 as O E2.

- Configure PC-C and PC-D IP settings

PC-C (Desktop > IP Configuration)
• IP: 192.168.10.10
• Mask: 255.255.255.0
• Gateway: 192.168.10.1

PC-D (Desktop > IP Configuration)
• IP: 192.168.20.10
• Mask: 255.255.255.0
• Gateway: 192.168.20.1

- Test connectivity

PC-A (Command Prompt)

ping 192.168.20.10

PC-D (Command Prompt)

ping 192.168.1.10

End of Lab Output Expectations
• Before redistribution: process 1 does not learn 192.168.10.0/24 or 192.168.20.0/24.
• After redistribution: routing tables show O E2 routes.
• PC-A can ping PC-D, and PC-D can ping PC-A (full reachability).