# OSPF Configuration & Debugging Lab

Topology
• 2x Routers: RouterA, RouterB
• 2x PCs: PC-A, PC-B
• Point-to-Point link: RouterA G0/0 ↔ RouterB G0/0 (10.0.0.0/30)
• LAN A: RouterA G0/1 (192.168.1.0/24)
• LAN B: RouterB G0/1 (192.168.2.0/24)

- Configure RouterA interfaces

RouterA

Router> no
Router> enable
Router# configure terminal

Router(config)# interface gigabitEthernet0/0
Router(config-if)# ip address 10.0.0.1 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1
Router(config-if)# ip address 192.168.1.1 255.255.255.0
Router(config-if)# no shutdown
Router(config-if)# exit

- Configure RouterB interfaces

RouterB

Router> no
Router> enable
Router# configure terminal

Router(config)# interface gigabitEthernet0/0
Router(config-if)# ip address 10.0.0.2 255.255.255.252
Router(config-if)# no shutdown
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1
Router(config-if)# ip address 192.168.2.1 255.255.255.0
Router(config-if)# no shutdown
Router(config-if)# exit

- Set hostnames + Loopbacks (stable router IDs)

RouterA

Router(config)# hostname RouterA
RouterA(config)# interface loopback0
RouterA(config-if)# ip address 1.1.1.1 255.255.255.255
RouterA(config-if)# exit

RouterB

Router(config)# hostname RouterB
RouterB(config)# interface loopback0
RouterB(config-if)# ip address 2.2.2.2 255.255.255.255
RouterB(config-if)# exit

- Start OSPF process 1 and set router-id

RouterA

RouterA(config)# router ospf 1
RouterA(config-router)# router-id 1.1.1.1
RouterA(config-router)# exit

RouterB

RouterB(config)# router ospf 1
RouterB(config-router)# router-id 2.2.2.2
RouterB(config-router)# exit

If router-id doesn’t reflect immediately

RouterA# clear ip ospf process
RouterB# clear ip ospf process

Verify router IDs

RouterA# show ip ospf
RouterB# show ip ospf

- Advertise networks in Area 0

RouterA

RouterA# configure terminal
RouterA(config)# router ospf 1
RouterA(config-router)# network 10.0.0.0 0.0.0.3 area 0
RouterA(config-router)# network 192.168.1.0 0.0.0.255 area 0
RouterA(config-router)# end

RouterB

RouterB# configure terminal
RouterB(config)# router ospf 1
RouterB(config-router)# network 10.0.0.0 0.0.0.3 area 0
RouterB(config-router)# network 192.168.2.0 0.0.0.255 area 0
RouterB(config-router)# end

- Verify neighbors and routes

Neighbors

RouterA# show ip ospf neighbor
RouterB# show ip ospf neighbor

Routing table

RouterA# show ip route
RouterB# show ip route

Expected
• RouterA learns 192.168.2.0/24 as O route via 10.0.0.2
• RouterB learns 192.168.1.0/24 as O route via 10.0.0.1

- Configure PCs and test ping

PC-A (Desktop > IP Configuration)
• IP: 192.168.1.10
• Mask: 255.255.255.0
• Gateway: 192.168.1.1

PC-B (Desktop > IP Configuration)
• IP: 192.168.2.10
• Mask: 255.255.255.0
• Gateway: 192.168.2.1

PC-A (Command Prompt)

ping 192.168.2.10

- Create area mismatch on RouterB (break adjacency)

RouterB

RouterB# show running-config
RouterB# configure terminal
RouterB(config)# router ospf 1
RouterB(config-router)# no network 10.0.0.0 0.0.0.3 area 0
RouterB(config-router)# network 10.0.0.0 0.0.0.3 area 1
RouterB(config-router)# end

Expected
• Neighbor goes from FULL to DOWN
• Message shows area mismatch / invalid packet

- Enable OSPF debugging

RouterA

RouterA# debug ip ospf events

RouterB

RouterB# debug ip ospf events

- Fix RouterB area back to 0 (restore adjacency)

RouterB

RouterB# configure terminal
RouterB(config)# router ospf 1
RouterB(config-router)# no network 10.0.0.0 0.0.0.3 area 1
RouterB(config-router)# network 10.0.0.0 0.0.0.3 area 0
RouterB(config-router)# end

Expected
• Neighbor state transitions: LOADING → FULL
• Debug shows hello processing + DR/BDR messages

Stop debugging

RouterA# undebug all
RouterB# undebug all

- Final verification

RouterA# show ip ospf
RouterA# show ip ospf neighbor
RouterB# show ip ospf
RouterB# show ip ospf neighbor

End of Lab Output Expectations
• OSPF neighbors reach FULL state in area 0
• O routes appear in both routing tables
• Area mismatch breaks adjacency immediately
• debug ip ospf events shows why adjacency fails and how it restores