SSH Configuration LAB

Topology 
• 1x Router (2911) — R1
• 1x PC — PC1
• Link: PC1 FastEthernet0 ↔ R1 GigabitEthernet0/0

- Cable the devices (Copper Straight-Through)
Connect these ports:
• PC1 Fa0 → R1 Gi0/0

- Configure IP addressing (same subnet 192.168.1.0/24)

PC1 (Desktop > IP Configuration)
• IP: 192.168.1.2
• Mask: 255.255.255.0
• Gateway: 192.168.1.1

R1

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# end

- Verify connectivity (ping)

PC1 (Command Prompt)

PC1> ping 192.168.1.1

- Configure SSH on R1

R1

Router# configure terminal

Router(config)# hostname R1

R1(config)# ip domain-name SSHLAB

R1(config)# username admin privilege 15 secret cisco123

R1(config)# crypto key generate rsa

When prompted for key size:
• Enter 1024

Continue configuration:

R1(config)# line vty 0 4

R1(config-line)# transport input ssh

R1(config-line)# login local

R1(config-line)# exit

R1(config)# ip ssh version 2

R1(config)# end

- Save configuration

R1

R1# write memory

- Test SSH using PC1 (Command Prompt)

PC1 (Command Prompt)

PC1> ssh -l admin 192.168.1.1

When prompted, enter password:
• cisco123

After login, run verification commands:

R1# show clock

R1# show running-config

R1# exit

End of Lab Output Expectations
• PC1 can ping 192.168.1.1 successfully.
• SSH version 2 is enabled on R1.
• VTY lines accept SSH only and use local user authentication.
• PC1 can SSH into R1 using username admin and password cisco123.