# CDP & LLDP Configuration LAB

Topology 
• 2x Switches (2960) — Switch0, Switch1
• 1x Router (2911) — Router0

- Cable the devices (Copper Straight-Through)
Connect these ports:
• Switch0 Fa0/1 → Switch1 Fa0/1
• Switch1 Fa0/2 → Router0 Gi0/0

- Bring up interfaces (no shutdown)

Switch0

Switch0> enable

Switch0# configure terminal

Switch0(config)# interface range fa0/1 - 2

Switch0(config-if-range)# no shutdown

Switch0(config-if-range)# exit

Switch1

Switch1> enable

Switch1# configure terminal

Switch1(config)# interface range fa0/1 - 2

Switch1(config-if-range)# no shutdown

Switch1(config-if-range)# exit

Switch1(config)# end

Router0

Router0> enable

Router0# configure terminal

Router0(config)# interface gigabitEthernet0/0

Router0(config-if)# no shutdown

Router0(config-if)# exit

Router0(config)# end

- Enable CDP (global + interface)

Switch0

Switch0# configure terminal

Switch0(config)# cdp run

Switch0(config)# interface fastEthernet0/1

Switch0(config-if)# cdp enable

Switch0(config-if)# exit

Switch0(config)# end

Switch1

Switch1# configure terminal

Switch1(config)# cdp run

Switch1(config)# interface range fa0/1 - 2

Switch1(config-if-range)# cdp enable

Switch1(config-if-range)# exit

Switch1(config)# end

Router0

Router0# configure terminal

Router0(config)# interface gigabitEthernet0/0

Router0(config-if)# cdp enable

Router0(config-if)# exit

Router0(config)# end

- Verify CDP neighbors

Switch0

Switch0# show cdp neighbors

Switch1

Switch1# show cdp neighbors

Switch1# show cdp neighbors detail

Switch1# show cdp interface

What to observe (CDP)
• Switch0 should discover Switch1 on Fa0/1
• Switch1 should discover two neighbors: Switch0 (Fa0/1) and Router0 (Fa0/2)
• “show cdp neighbors detail” gives more information like platform and IOS
• “show cdp interface” shows CDP status per interface and timers

- Enable LLDP (global + interface)

Switch0

Switch0# configure terminal

Switch0(config)# lldp run

Switch0(config)# interface fa0/1

Switch0(config-if)# lldp receive

Switch0(config-if)# lldp transmit

Switch0(config-if)# exit

Switch0(config)# end

Switch1

Switch1# configure terminal

Switch1(config)# lldp run

Switch1(config)# interface range fa0/1 - 2

Switch1(config-if-range)# lldp transmit

Switch1(config-if-range)# lldp receive

Switch1(config-if-range)# exit

Switch1(config)# end

- Verify LLDP neighbors before enabling router LLDP

Switch1

Switch1# show lldp neighbors

What to observe (before router LLDP)
• Only Switch0 appears because LLDP is not yet enabled on Router0
• LLDP requires both ends to participate for neighbor discovery

- Enable LLDP on Router0

Router0

Router0# configure terminal

Router0(config)# lldp run

Router0(config)# interface gigabitEthernet0/0

Router0(config-if)# lldp transmit

Router0(config-if)# lldp receive

Router0(config-if)# exit

Router0(config)# end

- Verify LLDP neighbors after enabling router LLDP

Switch1

Switch1# show lldp neighbors

Switch1# show lldp neighbors detail

What to observe (LLDP)
• Switch1 now discovers Router0 on Fa0/2
• “show lldp neighbors detail” shows system name, system description, port description, and capabilities

End of Lab Output Expectations
• CDP shows Cisco neighbors automatically (Switch0 ↔ Switch1 ↔ Router0)
• LLDP shows neighbors only after LLDP is enabled on both ends of each link
• “show cdp neighbors / detail / interface” and “show lldp neighbors / detail” confirm device IDs, ports, capabilities, and platform