## Etherchannel Configuration LAB

Topology
• 2x Multilayer Switches — MLS1, MLS2
• Layer 2 links (initial):
• MLS1 Fa0/1 ↔ MLS2 Fa0/1
• MLS1 Fa0/2 ↔ MLS2 Fa0/2
• Layer 3 links (later):
• MLS1 Gi0/1 ↔ MLS2 Gi0/1 
• MLS1 Gi0/2 ↔ MLS2 Gi0/2

- Cable the devices (Copper Straight-Through)
Connect these ports:
• MLS1 Fa0/1 → MLS2 Fa0/1
• MLS1 Fa0/2 → MLS2 Fa0/2

- Verify STP behavior before EtherChannel (one link blocks on non-root)

MLS1

MLS1# show spanning-tree

MLS2

MLS2# show spanning-tree

What to observe
• On MLS2, one of the parallel FastEthernet links may show blocking due to STP.

- Enable IP routing on both switches (required for Layer 3 EtherChannel later)

MLS1

MLS1# configure terminal

MLS1(config)# ip routing

MLS1(config)# end

MLS2

MLS2# configure terminal

MLS2(config)# ip routing

MLS2(config)# end

- Configure Layer 2 EtherChannel using LACP (Fa0/1–Fa0/2)

MLS1 (create Port-Channel 1 with LACP active)

MLS1# configure terminal

MLS1(config)# interface range fa0/1 - 2

MLS1(config-if-range)# channel-group 1 mode active

MLS1(config-if-range)# exit

MLS1(config)# interface port-channel 1

MLS1(config-if)# switchport trunk encapsulation dot1q

MLS1(config-if)# switchport mode trunk

MLS1(config-if)# end

MLS2 (create Port-Channel 1 with LACP active)

MLS2# configure terminal

MLS2(config)# interface range fa0/1 - 2

MLS2(config-if-range)# channel-group 1 mode active

MLS2(config-if-range)# exit

MLS2(config)# interface port-channel 1

MLS2(config-if)# switchport trunk encapsulation dot1q

MLS2(config-if)# switchport mode trunk

MLS2(config-if)# end

- Verify Layer 2 EtherChannel

MLS1

MLS1# show etherchannel summary

MLS1# show spanning-tree

MLS2

MLS2# show etherchannel summary

MLS2# show spanning-tree

What to observe
• Port-Channel1 is formed and shows bundled ports (Fa0/1 and Fa0/2).
• STP now treats the EtherChannel as one logical interface (Port-Channel1).

- Add Layer 3 links (Copper Straight-Through)
Connect these ports:
• MLS1 Gi0/1 → MLS2 Gi0/1
• MLS1 Gi0/2 → MLS2 Gi0/2

- Configure Layer 3 EtherChannel using LACP (Gi0/1–Gi0/2)

MLS1 (Port-Channel 2 as routed interface)

MLS1# configure terminal

MLS1(config)# interface range g0/1 - 2

MLS1(config-if-range)# no switchport

MLS1(config-if-range)# channel-group 2 mode active

MLS1(config-if-range)# exit

MLS1(config)# interface port-channel 2

MLS1(config-if)# no switchport

MLS1(config-if)# ip address 10.0.0.1 255.255.255.0

MLS1(config-if)# end

MLS2 (Port-Channel 2 as routed interface)

MLS2# configure terminal

MLS2(config)# interface range g0/1 - 2

MLS2(config-if-range)# no switchport

MLS2(config-if-range)# channel-group 2 mode active

MLS2(config-if-range)# exit

MLS2(config)# interface port-channel 2

MLS2(config-if)# no switchport

MLS2(config-if)# ip address 10.0.0.2 255.255.255.0

MLS2(config-if)# end

- Verify Layer 3 EtherChannel and IP status

MLS1

MLS1# show ip interface brief

MLS1# show etherchannel summary

MLS2

MLS2# show ip interface brief

MLS2# show etherchannel summary

- Test connectivity across Layer 3 EtherChannel

MLS1

MLS1# ping 10.0.0.2

- Simulate link failure (Gigabit link) and verify no downtime

MLS2 (shutdown Gi0/1)

MLS2# configure terminal

MLS2(config)# interface g0/1

MLS2(config-if)# shutdown

MLS2(config-if)# end

MLS1 (ping again)

MLS1# ping 10.0.0.2

MLS2 (no shutdown Gi0/1)

MLS2# configure terminal

MLS2(config)# interface g0/1

MLS2(config-if)# no shutdown

MLS2(config-if)# end

- Simulate link failure (FastEthernet in Port-Channel1)

MLS2 (shutdown Fa0/2)

MLS2# configure terminal

MLS2(config)# interface fa0/2

MLS2(config-if)# shutdown

MLS2(config-if)# end

MLS1 (verify EtherChannel status)

MLS1# show etherchannel summary

- Simulate another failure (GigabitEthernet in Port-Channel2)

MLS2 (shutdown Gi0/2)

MLS2# configure terminal

MLS2(config)# interface g0/2

MLS2(config-if)# shutdown

MLS2(config-if)# end

MLS1 (verify EtherChannel status and ping)

MLS1# show etherchannel summary

MLS1# ping 10.0.0.2

- Final STP check

MLS1

MLS1# show spanning-tree

MLS2

MLS2# show spanning-tree