## Router-on-a stick

## Switch configuration

## Switch>enable

## Switch#config t

## Enter configuration commands, one per line.  End with CNTL/Z.

## Switch(config)#vlan 10

## Switch(config-vlan)#name sales

## Switch(config-vlan)#exit

## Switch(config)#vlan 20

## Switch(config-vlan)#name hr

## Switch(config-vlan)#exit

## Switch(config)#interface fastEthernet 0/1

## Switch(config-if)#switchport mode access

## Switch(config-if)#switchport access vlan 10

## Switch(config-if)#exit

## Switch(config)#interface fastEthernet 0/2

## Switch(config-if)#switchport mode access

## Switch(config-if)#switchport access vlan 20

## Switch(config-if)#exit

## Switch(config)#exit

## Switch#

## %SYS-5-CONFIG_I: Configured from console by console

## Switch#sh vlan brief

## VLAN Name                             Status    Ports

## ---- -------------------------------- --------- -------------------------------

## 1    default                          active    Fa0/3, Fa0/4, Fa0/5, Fa0/6

## Fa0/7, Fa0/8, Fa0/9, Fa0/10

## Fa0/11, Fa0/12, Fa0/13, Fa0/14

## Fa0/15, Fa0/16, Fa0/17, Fa0/18

## Fa0/19, Fa0/20, Fa0/21, Fa0/22

## Fa0/23, Fa0/24, Gig0/1, Gig0/2

## 10   sales                            active    Fa0/1

## 20   hr                               active    Fa0/2

## 1002 fddi-default                     active

## 1003 token-ring-default               active

## 1004 fddinet-default                  active

## 1005 trnet-default                    active

## Switch#config t

## Enter configuration commands, one per line.  End with CNTL/Z.

## Switch(config)#interface fastEthernet 0/24

## Switch(config-if)#switchport mode trunk

## Switch(config-if)#switchport trunk allowed vlan all

## Switch(config-if)#no shut

## Switch(config-if)#exit

## Switch(config)#exit

## Router Configuration

## Router>enable

## Router#config t

## Enter configuration commands, one per line.  End with CNTL/Z.

## Router(config)#interface gigabitEtherent 0/0/0

## ^

## % Invalid input detected at '^' marker.

## Router(config)#interface gigabitEthernet 0/0/0

## Router(config-if)#no shut

## Router(config-if)#

## %LINK-5-CHANGED: Interface GigabitEthernet0/0/0, changed state to up

## %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to up

## Router(config-if)#exit

## Router(config)#interface gigabitEthernet 0/0/0.10

## Router(config-subif)#

## %LINK-3-UPDOWN: Interface GigabitEthernet0/0/0.10, changed state to down

## %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0.10, changed state to up

## Router(config-subif)#encapsulation dot1Q 10

## Router(config-subif)#ip address 192.168.10.1 255.255.255.0

## Router(config-subif)#exit

## Router(config)#interface gigabitEthernet 0/0/0.20

## Router(config-subif)#

## %LINK-3-UPDOWN: Interface GigabitEthernet0/0/0.20, changed state to down

## Router(config-subif)#encapsulation dot1Q 20

## Router(config-subif)#ip address 192.168.20.1 255.255.255.0

## Router(config-subif)#exit

## Router(config)#exit

## Router#

## %SYS-5-CONFIG_I: Configured from console by console

## Router#sh ip interface brief

## Interface              IP-Address      OK? Method Status                Protocol

## GigabitEthernet0/0/0   unassigned      YES unset  up                    up

## GigabitEthernet0/0/0.10192.168.10.1    YES manual up                    up

## GigabitEthernet0/0/0.20192.168.20.1    YES manual up                    up

## GigabitEthernet0/0/1   unassigned      YES unset  administratively down down

## GigabitEthernet0/0/2   unassigned      YES unset  administratively down down

## Vlan1                  unassigned      YES unset  administratively down down

## Router#

## ********Switch Virtual interface

## Switch(config)#interface fastEthernet 0/1

## Switch(config-if)#switchport mode access

## Switch(config-if)#switchport access vlan 30

## Switch(config-if)#exit

## Switch(config)#interface fastEthernet 0/2

## Switch(config-if)#switchport mode access

## Switch(config-if)#switchport access vlan 40

## Switch(config-if)#exit

## Switch(config)#ip routing

## Switch(config)#interface vlan 30

## Switch(config-if)#ip address 192.168.30.1 255.255.255.0

## Switch(config-if)#no shutdown

## Switch(config-if)#exit

## Switch(config)#interface vlan 40

## Switch(config-if)#ip address 192.168.40.1 255.255.255.0

## Switch(config-if)#no shutdown

## InterVLAN Routing Comprehensive LAB(Trunk + VTP + Router Sub-Interfaces)

Topology
• Segment A (Router-on-a-Stick)
• Switch0 (2960)
• Router0
• PC0 (VLAN 10) and PC1 (VLAN 20)
• Segment B (SVI Method)
• Multilayer Switch0
• PC2 (VLAN 30) and PC3 (VLAN 40)
• New Link
• Switch0 Fa0/23 ↔ Multilayer Switch0 Fa0/23 (Trunk)

- Create a trunk link between Switch0 and Multilayer Switch0
Cable
• Use Copper Straight-Through
• Connect Switch0 Fa0/23 to Multilayer Switch0 Fa0/23

- Configure trunk on Switch0 (Fa0/23)

Switch0

Switch0> enable

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/23

Switch0(config-if)# switchport mode trunk

Switch0(config-if)# switchport trunk allowed vlan all

Switch0(config-if)# exit

Switch0(config)# end

- Configure trunk on Multilayer Switch0 (Fa0/23)

Multilayer Switch0

MultilayerSwitch0> enable

MultilayerSwitch0# configure terminal

MultilayerSwitch0(config)# interface fastEthernet0/23

MultilayerSwitch0(config-if)# switchport trunk encapsulation dot1Q

MultilayerSwitch0(config-if)# switchport mode trunk

MultilayerSwitch0(config-if)# switchport trunk allowed vlan all

MultilayerSwitch0(config-if)# exit

MultilayerSwitch0(config)# end

- Verify VLANs before VTP

Multilayer Switch0

MultilayerSwitch0# show vlan

Switch0

Switch0# show vlan

- Configure VTP on Switch0

Switch0

Switch0# show vtp status

Switch0# configure terminal

Switch0(config)# vtp domain TESTLAB

Switch0(config)# vtp version 2

Switch0(config)# end

- Configure VTP on Multilayer Switch0
Set same domain and version, keep mode as server.

Multilayer Switch0

MultilayerSwitch0# configure terminal

MultilayerSwitch0(config)# vtp domain TESTLAB

MultilayerSwitch0(config)# vtp version 2

MultilayerSwitch0(config)# end

MultilayerSwitch0# show vtp status

- Create VLAN 10 and VLAN 20 on Multilayer Switch0 (to propagate to Switch0)

Multilayer Switch0

MultilayerSwitch0# configure terminal

MultilayerSwitch0(config)# vlan 10

MultilayerSwitch0(config-vlan)# name Sales

MultilayerSwitch0(config-vlan)# exit

MultilayerSwitch0(config)# vlan 20

MultilayerSwitch0(config-vlan)# name HR

MultilayerSwitch0(config-vlan)# exit

MultilayerSwitch0(config)# end

Verify on Switch0 (VLANs should appear via VTP)

Switch0# show vlan

- Test connectivity (before adding router sub-interfaces)
PC0 (Command Prompt)

PC0> ping 192.168.30.10

Expected: Destination host unreachable (router not configured for VLAN 30/40 yet)

- Verify router configuration before adding new sub-interfaces

Router0

Router0> enable

Router0# show running-config

- Configure Router0 sub-interfaces for VLAN 30 and VLAN 40

Router0

Router0# configure terminal

Router0(config)# interface gigabitEthernet0/0/0.30

Router0(config-subif)# encapsulation dot1Q 30

Router0(config-subif)# ip address 192.168.30.1 255.255.255.0

Router0(config-subif)# exit

Router0(config)# interface gigabitEthernet0/0/0.40

Router0(config-subif)# encapsulation dot1Q 40

Router0(config-subif)# ip address 192.168.40.1 255.255.255.0

Router0(config-subif)# exit

Router0(config)# end

- Test connectivity between all PCs (after router sub-interfaces)

PC0

PC0> ping 192.168.30.10

PC0> ping 192.168.40.10

PC3

PC3> ping 192.168.20.10

PC3> ping 192.168.10.10

End of Lab Output Expectations
• Switch0 and Multilayer Switch0 share VLAN database using VTP (VLAN 10, 20, 30, 40).
• Trunk between Switch0 and Multilayer Switch0 carries all VLANs.
• Router0 has sub-interfaces for VLAN 10, 20, 30, 40.
• All PCs across both segments can ping each other successfully.