## Spanning Tree Protocol (STP) Comprehensive LAB

Topology 
• 3x Switches (2960) — Switch0, Switch1, Switch2 (triangle topology)
• 2x PCs — PC0 and PC1 (for convergence testing)
• Later: 1 extra Switch (for BPDU Guard trigger test) and 1 extra PC (for PortFast test)

- Configure hostnames and trunk links (VLAN 1 default STP)

Switch0

Switch0> enable

Switch0# configure terminal

Switch0(config)# hostname Switch0

Switch0(config)# interface range fastEthernet0/1 - 2

Switch0(config-if-range)# switchport mode trunk

Switch0(config-if-range)# no shutdown

Switch0(config-if-range)# exit

Switch0(config)# end

Switch1

Switch1> enable

Switch1# configure terminal

Switch1(config)# hostname Switch1

Switch1(config)# interface range fastEthernet0/1 - 2

Switch1(config-if-range)# switchport mode trunk

Switch1(config-if-range)# no shutdown

Switch1(config-if-range)# exit

Switch1(config)# end

Switch2

Switch2> enable

Switch2# configure terminal

Switch2(config)# hostname Switch2

Switch2(config)# interface range fastEthernet0/1 - 2

Switch2(config-if-range)# switchport mode trunk

Switch2(config-if-range)# no shutdown

Switch2(config-if-range)# exit

Switch2(config)# end

- Verify default STP behavior (VLAN 1)

Switch0

Switch0# show spanning-tree vlan 1

Switch1

Switch1# show spanning-tree vlan 1

Switch2

Switch2# show spanning-tree vlan 1

- Verify STP port roles (root + non-root)

Switch1 (root bridge ports should be designated/forwarding)

Switch1# show spanning-tree interface fastEthernet0/1 detail

Switch1# show spanning-tree interface fastEthernet0/2 detail

Switch2 (root port forwarding, alternate port blocking)

Switch2# show spanning-tree interface fastEthernet0/1 detail

Switch2# show spanning-tree interface fastEthernet0/2 detail

- Manually set root bridge and secondary root (VLAN 1)

Switch0 (make root for VLAN 1)

Switch0# configure terminal

Switch0(config)# spanning-tree vlan 1 priority 4096

Switch0(config)# end

Switch1 (make secondary root for VLAN 1)

Switch1# configure terminal

Switch1(config)# spanning-tree vlan 1 priority 8192

Switch1(config)# end

Verify

Switch0# show spanning-tree vlan 1

Switch1# show spanning-tree vlan 1

- Configure IP addresses on PCs and test basic connectivity

PC0 (Desktop > IP Configuration)
• IP: 192.168.1.1
• Mask: 255.255.255.0
• Gateway: leave blank

PC1 (Desktop > IP Configuration)
• IP: 192.168.1.2
• Mask: 255.255.255.0
• Gateway: leave blank

PC0 (Command Prompt)

PC0> ping 192.168.1.2

- Simulate classic STP convergence (shutdown a link)

Switch0 (prepare interface Fa0/1)

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/1

PC0 (run continuous ping)

PC0> ping 192.168.1.2 -n 50

Switch0 (shutdown the port)

Switch0(config-if)# shutdown

- Restore link and enable Rapid PVST

Switch0 (bring port back up)

Switch0(config-if)# no shutdown

Switch0(config-if)# end

Switch0 (enable Rapid PVST)

Switch0# configure terminal

Switch0(config)# spanning-tree mode rapid-pvst

Switch0(config)# end

Switch1

Switch1# configure terminal

Switch1(config)# spanning-tree mode rapid-pvst

Switch1(config)# end

Switch2

Switch2# configure terminal

Switch2(config)# spanning-tree mode rapid-pvst

Switch2(config)# end

- Test Rapid PVST convergence (shutdown again)

Switch0

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/1

PC0 (run continuous ping)

PC0> ping 192.168.1.2 -n 50

Switch0 (shutdown the port)

Switch0(config-if)# shutdown

Switch0 (bring port back up after test)

Switch0(config-if)# no shutdown

Switch0(config-if)# end

- Create VLAN 10 and VLAN 20 on all switches

Switch0

Switch0# configure terminal

Switch0(config)# vlan 10

Switch0(config-vlan)# exit

Switch0(config)# vlan 20

Switch0(config-vlan)# exit

Switch0(config)# end

Switch1

Switch1# configure terminal

Switch1(config)# vlan 10

Switch1(config-vlan)# exit

Switch1(config)# vlan 20

Switch1(config-vlan)# exit

Switch1(config)# end

Switch2

Switch2# configure terminal

Switch2(config)# vlan 10

Switch2(config-vlan)# exit

Switch2(config)# vlan 20

Switch2(config-vlan)# exit

Switch2(config)# end

- Assign access ports for VLAN testing

Switch0 (Fa0/3 → VLAN 10)

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/3

Switch0(config-if)# switchport mode access

Switch0(config-if)# switchport access vlan 10

Switch0(config-if)# end

PC0 (change IP)
• IP: 192.168.10.1
• Mask: 255.255.255.0

Switch1 (Fa0/3 → VLAN 20)

Switch1# configure terminal

Switch1(config)# interface fastEthernet0/3

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 20

Switch1(config-if)# end

PC1 (change IP)
• IP: 192.168.20.1
• Mask: 255.255.255.0

- Configure different root bridges per VLAN (Rapid PVST)

Switch1 (root for VLAN 10)

Switch1# configure terminal

Switch1(config)# spanning-tree vlan 10 priority 4096

Switch1(config)# end

Switch2 (root for VLAN 20)

Switch2# configure terminal

Switch2(config)# spanning-tree vlan 20 priority 4096

Switch2(config)# end

Verify

Switch2# show spanning-tree vlan 1

Switch2# show spanning-tree vlan 20

Switch1# show spanning-tree vlan 10

END HERE

- Configure PortFast + BPDU Guard (simulate rogue switch)

Switch0 (Fa0/4 PortFast + BPDU Guard)

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/4

Switch0(config-if)# switchport mode access

Switch0(config-if)# spanning-tree portfast

Switch0(config-if)# spanning-tree bpduguard enable

Switch0(config-if)# end

Lab action (trigger BPDU Guard)
• Add a new 2960 switch
• Connect new switch Fa0/1 → Switch0 Fa0/4
Expected: Fa0/4 goes into error-disabled state

- Recover Fa0/4 after BPDU Guard (Packet Tracer workaround)

Switch0

Switch0# configure terminal

Switch0(config)# default interface fastEthernet0/4

Switch0(config)# interface fastEthernet0/4

Switch0(config-if)# no spanning-tree bpduguard enable

Switch0(config-if)# shutdown

Switch0(config-if)# no shutdown

Switch0(config-if)# spanning-tree bpduguard enable

Switch0(config-if)# end

- Configure Root Guard (example on Switch0 Fa0/1)

Switch0

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/1

Switch0(config-if)# spanning-tree guard root

Switch0(config-if)# end

End of Lab Output Expectations
• Default STP elects a root bridge and blocks one redundant port in the triangle
• After setting priorities, Switch0 becomes root and Switch1 becomes secondary root (VLAN 1)
• Classic STP convergence causes noticeable ping drops after link failure
• Rapid PVST converges much faster with minimal packet loss
• VLAN 10 and VLAN 20 can have different root bridges (per-VLAN spanning tree)
• PortFast makes access ports forward immediately
• BPDU Guard error-disables the port when a switch is connected to a PortFast port
• Root Guard protects against an unexpected root bridge change