# VTP Comprehensive Lab (3 Switches)

Topology  
• 3x Switches (2960) — Switch0, Switch1, Switch2
• End devices are already connected
• IP addresses are already assigned to PCs (as per workspace table)

- Configure trunk links (allow all VLANs)

Switch0 (Fa0/24 trunk)

Switch0> enable

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/24

Switch0(config-if)# switchport mode trunk

Switch0(config-if)# switchport trunk allowed vlan all

Switch0(config-if)# end

Switch1 (Fa0/24 and Fa0/23 trunk)

Switch1> enable

Switch1# configure terminal

Switch1(config)# interface fastEthernet0/24

Switch1(config-if)# switchport mode trunk

Switch1(config-if)# switchport trunk allowed vlan all

Switch1(config-if)# exit

Switch1(config)# interface fastEthernet0/23

Switch1(config-if)# switchport mode trunk

Switch1(config-if)# switchport trunk allowed vlan all

Switch1(config-if)# end

Switch1# show interfaces trunk

Switch2 (disable DNS lookup + trunk Fa0/23)

Switch2> enable

Switch2# configure terminal

Switch2(config)# no ip domain-lookup

Switch2(config)# interface fastEthernet0/23

Switch2(config-if)# switchport mode trunk

Switch2(config-if)# switchport trunk allowed vlan all

Switch2(config-if)# end

Switch2# show interfaces trunk

- Configure VTP domain and password

Switch0 (set domain + password, verify)

Switch0# configure terminal

Switch0(config)# vtp domain NetLab

Switch0(config)# vtp password ccna123

Switch0(config)# end

Switch0# show vtp status

Switch1 (set password, verify)

Switch1# configure terminal

Switch1(config)# vtp password ccna123

Switch1(config)# end

Switch1# show vtp status

Switch2 (domain is null until configured, then set domain + password)

Switch2# show vtp status

Switch2# configure terminal

Switch2(config)# vtp domain NetLab

Switch2(config)# vtp password ccna123

Switch2(config)# end

Switch2# show vtp status

- Set hostnames and VTP modes

Switch0 (Server)

Switch0# configure terminal

Switch0(config)# hostname Switch0

Switch0(config)# vtp mode server

Switch0(config)# end

Switch0# show vtp status

Switch1 (Client)

Switch1# configure terminal

Switch1(config)# hostname Switch1

Switch1(config)# vtp mode client

Switch1(config)# end

Switch1# show vtp status

Switch2 (Transparent)

Switch2# configure terminal

Switch2(config)# hostname Switch2

Switch2(config)# vtp mode transparent

Switch2(config)# end

Switch2# show vtp status

- Set VTP version 2

Switch0

Switch0# configure terminal

Switch0(config)# vtp version 2

Switch0(config)# end

Switch0# show vtp status

Switch1 (verify it inherited version 2)

Switch1# show vtp status

Switch2

Switch2# configure terminal

Switch2(config)# vtp version 2

Switch2(config)# end

Switch2# show vtp status

- Create VLANs on server and verify propagation

Switch0 (create VLAN 10/20/30 with names)

Switch0# configure terminal

Switch0(config)# vlan 10

Switch0(config-vlan)# name Sales

Switch0(config-vlan)# exit

Switch0(config)# vlan 20

Switch0(config-vlan)# name HR

Switch0(config-vlan)# exit

Switch0(config)# vlan 30

Switch0(config-vlan)# name IT

Switch0(config-vlan)# exit

Switch0(config)# end

Switch0# show vlan brief

Switch1 (verify VLANs propagated)

Switch1# show vlan brief

Switch2 (transparent does not synchronize VLANs)

Switch2# show vlan brief

- Assign VLANs to PC ports

Switch0 (Fa0/1 → VLAN 10)

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/1

Switch0(config-if)# switchport mode access

Switch0(config-if)# switchport access vlan 10

Switch0(config-if)# end

Switch1 (Fa0/2 → VLAN 20)

Switch1# configure terminal

Switch1(config)# interface fastEthernet0/2

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 20

Switch1(config-if)# end

Switch2 (Fa0/3 → VLAN 30; creates VLAN locally if missing)

Switch2# configure terminal

Switch2(config)# interface fastEthernet0/3

Switch2(config-if)# switchport mode access

Switch2(config-if)# switchport access vlan 30

Switch2(config-if)# end

Switch2# show vlan brief

- Ping test (VLAN isolation)

PC0 (Command Prompt)

PC0> ping 192.168.20.10

What to observe
• Ping fails because inter-VLAN routing is not configured.

- Check revision numbers

Switch0# show vtp status

Switch1# show vtp status

Switch2# show vtp status

What to observe
• Switch0 and Switch1 show the same revision number.
• Switch2 stays at revision 0 because it is transparent.

- Change Switch1 to server and create VLAN 50

Switch1

Switch1# configure terminal

Switch1(config)# vtp mode server

Switch1(config)# vlan 50

Switch1(config-vlan)# name Finance

Switch1(config-vlan)# end

Switch1# show vtp status

Switch1# show vlan brief

Switch0 (verify VLAN 50 propagated)

Switch0# show vlan brief

- Delete VLANs on Switch0 and observe synchronization behavior

Switch0

Switch0# configure terminal

Switch0(config)# no vlan 10

Switch0(config)# no vlan 20

Switch0(config)# no vlan 30

Switch0(config)# end

Switch0# show vlan brief

Switch2 (transparent does not synchronize VLAN 50)

Switch2# show vlan brief

- Create VLAN 60 on Switch0 and verify propagation

Switch0

Switch0# configure terminal

Switch0(config)# vlan 60

Switch0(config-vlan)# exit

Switch0(config)# end

Switch0# show vlan brief

Switch1

Switch1# show vlan brief

- Change Switch0 to client (revision does not reset)

Switch0

Switch0# configure terminal

Switch0(config)# vtp mode client

Switch0(config)# end

Switch0# show vtp status

- Reset revision number on Switch1 (delete vlan.dat + reload)

Switch1

Switch1# delete vlan.dat

Switch1# reload

After reload

Switch1> enable

Switch1# show vtp status

- Create VLAN 70 on Switch1 and verify behavior on Switch0

Switch1

Switch1# configure terminal

Switch1(config)# vlan 70

Switch1(config-vlan)# name Test-VLAN

Switch1(config-vlan)# end

Switch1# show vlan brief

Switch1# show vtp status

Switch0

Switch0# show vlan brief

Switch0# show vtp status

What to observe
• VLAN 70 may not appear on Switch0 if Switch0 has a higher revision number.

- Reset revision number on Switch0 (delete vlan.dat + reload)

Switch0

Switch0# delete vlan.dat

Switch0# reload

After reload

Switch0> enable

Switch0# show vtp status

Switch0# show vlan brief

- Create VLAN 80 on Switch1 and verify propagation to Switch0

Switch1

Switch1# configure terminal

Switch1(config)# vlan 80

Switch1(config-vlan)# name test-vlan2

Switch1(config-vlan)# end

Switch1# show vlan brief

Switch0

Switch0# show vlan brief