# VLAN Membership Lab (One Switch + 3 PCs, /26 Subnetting)

## Objective

- Create 3 VLANs (HR, Finance, IT) on a 2960 switch.

- Assign switch ports to VLANs.

- Assign IPs from different /26 subnets to each PC.

- Test that devices in different VLANs cannot ping without routing.

- Verify VLAN and port membership using show commands.

IP Plan (Class C 192.168.1.0/24 subnetted to /26)
Subnet mask for all PCs: 255.255.255.192 (/26)

- VLAN 2 (HR): 192.168.1.0/26 usable 192.168.1.1–192.168.1.62
HR PC IP: 192.168.1.10

- VLAN 3 (Finance): 192.168.1.64/26 usable 192.168.1.65–192.168.1.126
Finance PC IP: 192.168.1.70

- VLAN 4 (IT): 192.168.1.128/26 usable 192.168.1.129–192.168.1.190
IT PC IP: 192.168.1.130

Note: Default gateway is not required in this lab because we are not doing inter-VLAN routing.

- Topology Setup (Packet Tracer)
Devices

- 1x Switch (2960)

- 3x PCs

Rename PCs (Display Name)

- PC0 → HR

- PC1 → Finance

- PC2 → IT

Cabling (Copper Straight-Through)

- HR PC Fa0 → Switch Fa0/1

- Finance PC Fa0 → Switch Fa0/2

- IT PC Fa0 → Switch Fa0/3

- Switch Basic CLI Access + Hostname
Open the switch CLI and enter:

Switch> enable

Switch# configure terminal

Switch(config)# hostname DeptSwitch

DeptSwitch(config)# exit

DeptSwitch#

- Create VLANs (HR, Finance, IT)
From global configuration mode, create VLAN 2, 3, 4 and name them:

DeptSwitch# configure terminal

DeptSwitch(config)# vlan 2

DeptSwitch(config-vlan)# name HR

DeptSwitch(config-vlan)# exit

DeptSwitch(config)# vlan 3

DeptSwitch(config-vlan)# name Finance

DeptSwitch(config-vlan)# exit

DeptSwitch(config)# vlan 4

DeptSwitch(config-vlan)# name IT

DeptSwitch(config-vlan)# exit

- Assign Switch Ports to VLANs (Access Ports)
HR on Fa0/1 → VLAN 2
Finance on Fa0/2 → VLAN 3
IT on Fa0/3 → VLAN 4

DeptSwitch(config)# interface fastEthernet0/1

DeptSwitch(config-if)# switchport mode access

DeptSwitch(config-if)# switchport access vlan 2

DeptSwitch(config-if)# exit

DeptSwitch(config)# interface fastEthernet0/2

DeptSwitch(config-if)# switchport mode access

DeptSwitch(config-if)# switchport access vlan 3

DeptSwitch(config-if)# exit

DeptSwitch(config)# interface fastEthernet0/3

DeptSwitch(config-if)# switchport mode access

DeptSwitch(config-if)# switchport access vlan 4

DeptSwitch(config-if)# exit

DeptSwitch(config)# end

DeptSwitch#

- Configure IP Addresses on PCs (Desktop > IP Configuration)
HR PC

- IP: 192.168.1.10

- Mask: 255.255.255.192

Finance PC

- IP: 192.168.1.70

- Mask: 255.255.255.192

IT PC

- IP: 192.168.1.130

- Mask: 255.255.255.192

- Test Connectivity (Should Fail Between VLANs)
HR PC Command Prompt:

HR> ping 192.168.1.70

Finance PC Command Prompt:

Finance> ping 192.168.1.10

IT PC Command Prompt:

IT> ping 192.168.1.10

Expected Result

- All pings fail because the PCs are in different VLANs and there is no router / Layer 3 switch to route between VLANs.

- This confirms VLANs isolate broadcast domains.

- Verification on the Switch

Check VLANs and port membership:

DeptSwitch# show vlan brief

Check port mode and assigned VLAN (example Fa0/1):

DeptSwitch# show interfaces fastEthernet0/1 switchport

(Repeat for Fa0/2 and Fa0/3 if needed.)

End Result Checklist

- VLAN 2 named HR has Fa0/1

- VLAN 3 named Finance has Fa0/2

- VLAN 4 named IT has Fa0/3

- PCs have correct /26 IPs

- Pings between VLANs fail (no inter-VLAN routing)