# Trunk Configuration Lab (VLANs + Trunks + Native VLAN + Troubleshooting)

Topology  
• 3x Switches (2960) — Switch0, Switch1, Switch2
• 4x PCs (PC0–PC3)
• Trunk links:
• Switch0 Fa0/24 ↔ Switch1 Fa0/24
• Switch1 Fa0/23 ↔ Switch2 Fa0/23

- Verify physical connectivity
• Wait until all links turn green.

- Create VLANs on each switch

Switch0 (create VLAN 10 and 20 only)

Switch0> enable

Switch0# configure terminal

Switch0(config)# vlan 10

Switch0(config-vlan)# exit

Switch0(config)# vlan 20

Switch0(config-vlan)# exit

Switch0(config)# end

Switch1 (create VLAN 10, 20, 30)

Switch1> enable

Switch1# configure terminal

Switch1(config)# vlan 10

Switch1(config-vlan)# exit

Switch1(config)# vlan 20

Switch1(config-vlan)# exit

Switch1(config)# vlan 30

Switch1(config-vlan)# exit

Switch1(config)# end

Switch1# show vlan

Switch2 (create VLAN 10, 20, 30)

Switch2> enable

Switch2# configure terminal

Switch2(config)# vlan 10

Switch2(config-vlan)# exit

Switch2(config)# vlan 20

Switch2(config-vlan)# exit

Switch2(config)# vlan 30

Switch2(config-vlan)# exit

Switch2(config)# end

Switch2# show vlan

- Configure access ports (PC-facing ports)

Switch0 (PC0 on Fa0/1 → VLAN 10)

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/1

Switch0(config-if)# switchport mode access

Switch0(config-if)# switchport access vlan 10

Switch0(config-if)# end

Switch0# show vlan

Switch1 (PC2 on Fa0/2 → VLAN 20)

Switch1# configure terminal

Switch1(config)# interface fastEthernet0/2

Switch1(config-if)# switchport mode access

Switch1(config-if)# switchport access vlan 20

Switch1(config-if)# end

Switch1# show vlan

Switch2 (PC1 on Fa0/1 → VLAN 10, PC3 on Fa0/2 → VLAN 30)

Switch2# configure terminal

Switch2(config)# interface fastEthernet0/1

Switch2(config-if)# switchport mode access

Switch2(config-if)# switchport access vlan 10

Switch2(config-if)# exit

Switch2(config)# interface fastEthernet0/2

Switch2(config-if)# switchport mode access

Switch2(config-if)# switchport access vlan 30

Switch2(config-if)# end

Switch2# show vlan

- Configure trunk links + allowed VLANs

Switch0 trunk (Fa0/24) + native VLAN 999

Switch0# configure terminal

Switch0(config)# interface fastEthernet0/24

Switch0(config-if)# switchport mode trunk

Switch0(config-if)# switchport trunk allowed vlan 10,20,30

Switch0(config-if)# switchport trunk native vlan 999

Switch0(config-if)# end

Switch1 trunk to Switch0 (Fa0/24) + native VLAN 999

Switch1# configure terminal

Switch1(config)# interface fastEthernet0/24

Switch1(config-if)# switchport mode trunk

Switch1(config-if)# switchport trunk allowed vlan 10,20,30

Switch1(config-if)# switchport trunk native vlan 999

Switch1(config-if)# exit

Switch1 trunk to Switch2 (Fa0/23) — no native VLAN command here

Switch1(config)# interface fastEthernet0/23

Switch1(config-if)# switchport mode trunk

Switch1(config-if)# switchport trunk allowed vlan 10,20,30

Switch1(config-if)# exit

Switch1(config)# end

Switch2 trunk (Fa0/23) — no native VLAN command yet (default native VLAN 1)

Switch2# configure terminal

Switch2(config)# interface fastEthernet0/23

Switch2(config-if)# switchport mode trunk

Switch2(config-if)# switchport trunk allowed vlan 10,20,30

Switch2(config-if)# end

- Configure PC IP addresses (Desktop > IP Configuration)

PC0
• IP: 192.168.10.1
• Mask: 255.255.255.0

PC1
• IP: 192.168.10.2
• Mask: 255.255.255.0

PC2
• IP: 192.168.20.1
• Mask: 255.255.255.0

PC3
• IP: 192.168.30.1
• Mask: 255.255.255.0

- Verify trunks and VLANs

Switch0

Switch0# show interfaces trunk

Switch1

Switch1# show interfaces trunk

Switch1# show vlan brief

Switch1# show interfaces fastEthernet0/24 switchport

Switch2

Switch2# show interfaces trunk

What to observe
• Switch0 and Switch1 show native VLAN as 999 on Fa0/24.
• Switch2 shows native VLAN as 1 on Fa0/23 (mismatch warning appears).
• Trunks show VLANs 10,20,30 allowed.

- Test connectivity (pings)

PC0 → PC1 (same VLAN 10, should succeed)

PC0> ping 192.168.10.2

PC2 → PC3 (VLAN 20 to VLAN 30, should fail; no routing)

PC2> ping 192.168.30.1

PC3 → PC0 (should fail; VLAN 30 missing on Switch0 at this stage)

PC3> ping 192.168.10.1

- Troubleshoot native VLAN mismatch (fix on Switch2)

Switch2

Switch2# show interfaces trunk

Switch2# configure terminal

Switch2(config)# interface fastEthernet0/23

Switch2(config-if)# switchport trunk native vlan 999

Switch2(config-if)# end

Switch2# show interfaces trunk

- Fix missing VLAN 30 on Switch0

Switch0

Switch0# configure terminal

Switch0(config)# vlan 30

Switch0(config-vlan)# exit

Switch0(config)# end

Switch0# show vlan brief

- Retest and final result

PC3 → PC0 (still fails)

PC3> ping 192.168.10.1

What to observe
• Now trunking + VLAN 30 are correct across switches.
• Ping still fails because VLAN 10 and VLAN 30 are different VLANs.
• Inter-VLAN communication requires a router or multilayer switch (not configured here).

End of Lab Output Expectations
• VLANs 10/20/30 exist on all switches after the fix.
• Trunks carry VLANs 10/20/30 and native VLAN is consistent (999) after the fix.
• PC0 can ping PC1 (same VLAN 10 across switches).
• PC2 cannot ping PC3 (different VLANs, no routing).
• PC3 cannot ping PC0 (different VLANs, no routing), even after fixing VLAN 30 and native VLAN.