# Understanding Switch Basic Configuration (Cisco 2960)

- Open the switch CLI

- Click the switch → CLI tab → press Enter until you see the prompt.

- Verify the switch IOS and hardware (User EXEC mode)

Switch> show version

- Enter Privileged EXEC mode (admin mode)

Switch> enable

Switch#

- Enter Global Configuration mode

Switch# configure terminal

Switch(config)#

- Set the hostname

Switch(config)# hostname Access01

Access01(config)#

- Configure the MOTD banner

Access01(config)# banner motd "The switch is restricted"

- Secure console access (Console line configuration)

Access01(config)# line console 0

Access01(config-line)# password Cisco123

Access01(config-line)# login

Access01(config-line)# exit

Access01(config)#

- Secure remote access (VTY line configuration)

Access01(config)# line vty 0 4

Access01(config-line)# password Remote123

Access01(config-line)# login

Access01(config-line)# exit

Access01(config)#

- Secure privileged EXEC access (enable password + enable secret)

Access01(config)# enable password Class123

Access01(config)# enable secret Secure123

- Exit to Privileged EXEC mode

Access01(config)# exit

Access01#

- Use help to explore commands

Access01# show ?

Access01# show interfaces ?

- Verify configuration and status (Privileged EXEC mode)

Access01# show running-config

Access01# show startup-config

Access01# show version

Access01# show history

Access01# show ip interface brief

Access01# show interfaces

- Save configuration (RAM to NVRAM)

Access01# copy running-config startup-config

- When prompted for destination filename, press Enter.

- Undo example: remove hostname using “no” (Global Configuration mode)

Access01# configure terminal

Access01(config)# no hostname

Switch(config)# exit

Switch#

- Check and set the system clock (Privileged EXEC mode)

Switch# show clock

Switch# clock set 09:09:09 23 September 2025

Switch# show clock

- Reset the switch (erase saved config and reload)

- Use only if you want a fresh start.

Switch# erase startup-config

Switch# reload