# Switching Concepts Lab – MAC Learning (Packet Tracer)

Topology

- 1x Switch (2960)

- 4x PCs (PC0–PC3)

- 1x Server (DHCP Server)

- Cable the devices (Copper Straight-Through)
Connect these ports:

- PC0 Fa0 → Switch Fa0/1

- PC1 Fa0 → Switch Fa0/2

- PC2 Fa0 → Switch Fa0/3

- PC3 Fa0 → Switch Fa0/4

- Server Fa0 → Switch Fa0/5

- Configure IP addressing (PC0 and PC1 static)
PC0 (Desktop > IP Configuration)

- IP: 192.168.1.10

- Mask: 255.255.255.0

- Gateway: 192.168.1.1

PC1 (Desktop > IP Configuration)

- IP: 192.168.1.20

- Mask: 255.255.255.0

- Gateway: 192.168.1.1

Leave PC2 and PC3 for DHCP (do not set static IPs).

- Configure the DHCP Server (static IP + DHCP service)
Server (Desktop > IP Configuration)

- IP: 192.168.1.100

- Mask: 255.255.255.0

- Gateway: 192.168.1.1

Server (Services > DHCP)

- Interface: FastEthernet0

- DHCP: On

- Default Gateway: 192.168.1.1

- DNS Server: 8.8.8.8

- Start IP Address: 192.168.1.30

- Maximum Users: 5

- Save

- Get DHCP addresses on PC2 and PC3
PC2 (Desktop > IP Configuration)

- Select: DHCP
Expected: gets 192.168.1.30

PC3 (Desktop > IP Configuration)

- Select: DHCP
Expected: gets 192.168.1.31

- Generate traffic to trigger MAC learning (pings)
PC0 (Desktop > Command Prompt)

PC0> ping 192.168.1.20

PC0> ping 192.168.1.30

PC0> ping 192.168.1.31

PC2 (Desktop > Command Prompt)

PC2> ping 192.168.1.10

PC2> ping 192.168.1.20

- View the MAC address table on the switch
Open Switch CLI and run:

Switch> enable

Switch# show mac address-table

Switch# show mac address-table dynamic

Switch# show mac address-table ?

Switch# show mac address-table static

What to observe

- You should see learned MAC addresses mapped to ports Fa0/1–Fa0/5.

- Dynamic entries appear after the ping traffic.

- Configure a Static MAC address entry (example: PC0 on Fa0/1)
First, copy the MAC address of PC0 from Packet Tracer (PC0 > Config > FastEthernet0 or PC0 details).
Then apply it on the switch:

Switch# configure terminal

Switch(config)# mac address-table static <PC0_MAC_ADDRESS> vlan 1 interface fastEthernet0/1

Switch(config)# end

Switch# show mac address-table static

- Clear the MAC address table and verify relearning
Clear learned entries:

Switch# clear mac address-table

Switch# show mac address-table

Generate traffic again (PC0 ping PC1):

PC0> ping 192.168.1.20

Check table again:

Switch# show mac address-table

- MAC aging time (real switch command)
On real Cisco switches you can set aging time like this:

Switch# configure terminal

Switch(config)# mac address-table aging-time <SECONDS>

Switch(config)# end

Note: Packet Tracer does not support changing or viewing MAC aging time. It uses a fixed default behavior.

End of Lab Output Expectations

- DHCP works: PC2 and PC3 receive IPs from the server.

- MAC table shows dynamic entries after ping traffic.

- Static MAC entry stays associated to the port until removed manually.

- Clearing the table removes entries, and traffic causes relearning again.